Cloud Computing Assignment: VirtualBox-Based Microservice Deployment

Objective

The objective of this assignment is to create and configure multiple Virtual Machines (VMs) using VirtualBox, establish a network between them, and deploy a simple Flask-based microservice-based application across the connected VMs to send data from one VM to another.

Github link

https://github.com/AbhilashAgarwalIITJ/VCC_Assignment1_VirtualBox_VM/tree/main

Video link (google drive)

Step-by-Step Implementation Guide

1. Installing VirtualBox and Creating Multiple VMs

Installation of VirtualBox

- Downloaded VirtualBox from the official website: https://www.virtualbox.org/.

- Installed VirtualBox following the installation steps for my Windows 11 operating system.

- Creating Virtual Machines (VMs)

- Opened VirtualBox and clicked New to create a new VM.

- Created VM (VM1), selected Linux (lubuntu-24.04.1-desktop-amd64.iso) as the OS, and allocated 2GB RAM and 25GB storage, 2 core CPU.

- Repeat the above steps to create a second VM (VM2).

2. Configuring Network Settings

- Configured the Network Adapter:

  - Go to VirtualBox->Tools > Network

  - Selected Host-only Networks and clicked on Create to create a new Host-only Networks - VirtualBox Host-only Ethernet Adaptor

  - Clicked on DHCP tab and selected the Enable Server check box

- Repeat steps for both VM1 and VM2

  - Start VM1 and log in.

  - Go to Setting->Network->Adaptor1

  - Select Attached to - > Host-Only Adaptor and pick VirtualBox Host-only Ethernet Adaptor

- Got IP address of of VM1 - 192.168.187.3

- Got IP address of of VM2 - 192.168.187.4

- Test connectivity between VMs:

ping 192.168.187.4  # From VM1

ping 192.168.187.3  # From VM2

3. Deploying a Simple Flask-Based Microservice Application

3.1. Enable Internet on VM1 and VM2

  - Go to Setting->Network->Adaptor1

  - Select Attached to - >NAT

3.2 Setting up Flask on VM2 (Server)

- Install Python and Flask:

sudo apt update

sudo apt install python3 python3-pip -y

pip3 install flask

- Created a flask app - app.py:

from flask import Flask, jsonify

app = Flask(__name__)

@app.route('/message', methods=['GET'])

def send_message():

return jsonify({"message": "Hello from VM2!"})

if __name__ == '__main__':

app.run(host='0.0.0.0', port=5000)

- Go to Setting->Network->Adaptor1

- Attached to - > Host-Only Adaptor and pick VirtualBox Host-only Ethernet Adaptor

- Run this flask app:

python3 app.py

3.3. Setting up a Client on VM1

- Install curl and requests:

sudo apt install curl -y

pip3 install requests

- Created a client script (client.py) to request data from VM2:

import requests

response = requests.get("http://192.168.187.3:5000/message")

print("Response from VM1:", response.json())

- Go to Setting->Network->Adaptor1

- Attached to - > Host-Only Adaptor and pick VirtualBox Host-only Ethernet Adaptor

- Run the client script:

python3 client.py

4. Testing and Verification

- Run the Flask server on VM2.

- Run the client script on VM1.

- Client running on VM1 received the message from the server VM2.

VM1

IP : 192.168.187.3

Python client calling message REST API hosted on VM1

VM2

IP : 192.168.187.4

Flask Server running on 5000 port, exposed message REST API

Architecture Diagram

6. Conclusion

This assignment successfully demonstrates the creation and configuration of multiple Virtual Machines using VirtualBox, the establishment of an internal network between the VMs, and the deployment of a microservice-based Flask application. The communication between VMs is tested by sending a message from VM2 to VM1.